HOW TO MAKE, FIT AND MAINTAIN A PACKSADDLE

A Donkey Sanctuary harness resource

Guide to captions for the film

| TIME | CAPTION | SCREENSHOT |
| --- | --- | --- |
| 00:10 | Donkeys and mules are used to carry cargo in many parts of the world |  |
| 00:16 | These heavy loads are often placed directly on the donkey’s back or on ill-fitting packsaddles |  |
| 00:20 | They rub against the animal’s skin |  |
| 00:24 | Causing nasty and sometimes life-threatening wounds |  |
| 00:28 | A simple and inexpensive packsaddle can make a big difference in reducing the animal’s discomfort and wounds |  |

| 00:34 | It has a gullet space that protects the spine |  |
| --- | --- | --- |
| 00:34 | And bars on the side that help distribute the weight of the cargo evenly |  |
| 00:43 | Maiwa and Chemusa are going to make a packsaddle for the first time |  |
| 00:47 | They will need: - A tape measure - Scissors or a knife - A marking pen - A needle - Strong thread |  |
| 00:59 | A thick rod cut into a V shape on one end |  |
| 01:05 | - A sisal sack - Straw or hay - A bucket of water - A piece of blanket - Rope |  |
| 01:13 | Take the open, shorter end of the sack |  |

| 01:19 | Hem it shut using a whip stitch |  |
| --- | --- | --- |
| 01:24 | Once sewn shut your sack will look like this |  |
| 01:28 | Now open one side of the sack along its length. You can use your hands to do this |  |
| 01:37 | Once open, your sack will look like this |  |
| 01:42 | Find the mid-point of the sack by folding it into half lengthways |  |
| 01:47 | Draw a straight line across this midpoint |  |
| 01:51 | Measure and mark a point roughly 4 centimetres on either side of the midpoint |  |

| 01:59 | Draw a straight line across the sack on either side to mark these points |  |
| --- | --- | --- |
| 02:03 | Measure a 20 centimetres out from one of the 4 centimetre lines and mark the point |  |
| 02:07 | Repeat the same on the other side |  |
| 02:10 | Draw lines across the sack to mark both 20 centimetre points |  |
| 02:13 | Your sack will now have these sections |  |
| 02:15 | Panel |  |
| 02:17 | Side bar |  |

| 02:19 | Gullet |  |
| --- | --- | --- |
| 02:20 | Stitch along each of the 4 outer lines you have drawn |  |
| 02:24 | You don’t need to sew the midpoint marker |  |
| 02:28 | A simple running stitch is fine |  |
| 02:32 | This will make 5 separate pockets, 4 to stuff with straw and a space in the middle to go over the donkey’s spine |  |
| 02:43 | Before you start the next stage, it is worth sprinkling a little water over your pile of straw |  |
| 02:47 | Damp straw will bend and twist without snapping |  |

| 02:51 | Take a small bundle of straw, around a centimetre thick |  |
| --- | --- | --- |
| 02:57 | Twist and fold the straw and place it between the fork of the V shaped rod |  |
| 03:02 | Use the V shaped rod |  |
| 03:06 | Push it down firmly into the bottom corners |  |
| 03:13 | Keep adding bundles of straw carefully, placing each bundle separately to avoid lumps and hollows |  |
| 03:24 | Repeat this to get as much straw in the side bar pocket as possible |  |
| 03:31 | When it is fairly full, beat the pocket down very hard. Add more straw so that it is really firm |  |

| 03:36 | Repeat this to fill the second side bar pocket |  |
| --- | --- | --- |
| 03:43 | The outer panels should then be stuffed to one- third the thickness of the side bar pockets |  |
| 03:52 | Your packsaddle should now look like this |  |
| 03:57 | Reseal the open length of the sack a whip stitch |  |
| 04:03 | Take care that the straw doesn’t fall out while stitching |  |
| 04:09 | The outer panels need to be quilted so that the stuffing doesn’t settle at the bottom |  |
| 04:13 | Lay the pack flat and mark out points at a 5 centimetre distance from each other across the panel |  |

| 04:24 | Use quilting stitches to build a pattern of squares |  |
| --- | --- | --- |
| 04:31 | Make sure that the stuffing is evenly spread and goes right into the corners |  |
| 04:42 | Your packsaddle is now ready for some straps and loops to hold it in place |  |
| 04:46 | The loops will help tie the girth or bellyband, breeching and breast collar straps |  |
| 04:55 | Find a flat material for the girth or bellyband |  |
| 05:01 | In Kenya, two flat ropes are sewn together |  |
| 05:04 | If your donkey will be carrying loads up and down hills, your packsaddle will need: |  |

| 05:08 | A breast collar at the front |  |
| --- | --- | --- |
| 05:11 | And breeching at the back |  |
| 05:15 | Straps should be 6 – 10 centimetres wide |  |
| 05:18 | And made of natural material such as sisal, cotton or leather |  |
| 05:20 | Each strap must be lined with a soft material such as wool, denim or cotton |  |
| 05:26 | Your finished packsaddle should look similar to this |  |
| 05:30 | The loops can be stitched higher up the packsaddle, on top of the panels so that they are out of the way of the cargo |  |

| 05:34 | The packsaddle should fit comfortably on your donkey’s back |  |
| --- | --- | --- |
| 05:37 | And leave a gullet space that protects the spine even when the cargo is loaded |  |
| 05:41 | Since the straps are well covered with a cotton material, your donkey won’t develop wounds |  |
| 05:51 | Repair and Maintenance |  |
| 05:54 | If your packsaddle wears out, you can easily repair it |  |
| 05:58 | Cut a piece of material in the shape of the torn area |  |
| 06:01 | Place the patch inside and stitch it |  |
| 06:09 | Your packsaddle will be as good as new |  |

| 06:13 | If your donkey develops a wound, for example near the spine |  |
| --- | --- | --- |
| 06:15 | You should first treat the wound and rest the donkey until it is completely healed |  |
| 06:19 | If you do need to work the donkey during this time, make sure that the cargo does not push on the wound |  |
| 06:24 | To protect the wound, beat a depression into the stuffing on the underside of the packsaddle |  |
| 06:29 | Then sew a couple of stitches straight through the packsaddle to hold the depression in place |  |
| 06:34 | Pull the stitches tight to keep the packsaddle away from the wound |  |
| 06:39 | This will take the pressure off the wound and allow it to heal |  |

| 06:52 | Keep your packsaddle clean by regularly brushing it to remove dirt |  |
| --- | --- | --- |
| 06:59 | Hang it in a dry place for airing when not in use |  |